Guide Specification

WOOD GRILLE, SERIES 1000
STYLE: 1500 Baffle

PROJECT:

09 54 26 – LINEAR WOOD CEILINGS

PART 1 – GENERAL

1.1	SUMMARY

A.	Section includes:

1.	Concealed suspension system for Wood Baffle ceiling.

2.	Wood Baffle ceiling for concealed suspension system.

3.	Trim and accessories.

4.	Seismic restraints for suspended ceiling system.

1.2	RELATED WORK IN OTHER SECTIONS:

A.	Division 1 – “General Conditions” for substitution requests, submittals, etc.

B.	Division 9 – “Acoustic Ceilings.”

C.	Division 13 – “Integrated Assemblies.”

D.	Division 15 – “Mechanical” for work to be coordinated with ceiling.

E.	Division 16 – “Electrical” for light fixture coordination.

1.3	REFERENCES

A.	ASCE 7-10: Minimum Design Loads and Associated Criteria for Buildings and Other Structures; 2010.

B.	ASTM A 1023 / A 1023M : Standard Specification For Stranded Carbon Steel Wire Ropes For General Purposes; 2019

C. 	ASTM C 423: Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method; 1990.

D.	ASTM E 84: Standard Test Method for Surface Burning Characteristics of Building Materials; 1991.

E.	AWI (QSI): Architectural Woodwork Quality Standards Illustrated; 2003.

F.	CISCA: Ceiling Systems Handbook.

1.4	QUALITY ASSURANCE

A.	Manufacturer Qualifications: Manufacturers other than those listed in Paragraph 2.1 are required to submit for approval prior to bidding per Section One.

B.	Installer Qualifications: Engage an experienced Installer, approved by wood ceiling manufacturer, who has completed ceilings similar in species, design, and extent to that indicated for this Project and with a record of successful in-service performance.

C.	Inspection: All work must pass inspection and approval of architect, as well as the local codes and regulations or authorities having jurisdiction.

D.	Single-Source Responsibility for Wood Ceiling System: Obtain each type of Wood Baffle from a single fabricator, with in-house Shop Drawing capabilities, in-house assembly and finishing capabilities, and with resources to provide products of consistent quality in appearance and physical properties without delaying the project.

E.	Single-Source Responsibility for Suspension System: Obtain each type of suspension system from a single source with resources to provide products of consistent quality in appearance and physical properties without delaying project.

F.	Pre-Installation Conference: Conduct conference at Project site to comply with requirements of Division 1 Section "Project Meetings."

1.5	SUBMITTALS

A.	General: Submit each item in this Section according to the Conditions of the Contract and Division 1 Specification Sections.

B.	Product Data: For each type of product specified.

C.	Samples: For verification of each type of exposed finish required, prepared on samples of size indicated below. Where finishes involve normal color and texture variations, include sample sets showing the range of variations expected.

1.	12” x 15-1/2” samples of each baffle type, pattern, and color.

1.6	SHOP DRAWINGS & COORDINATION WITH OTHER TRADES

A.	Shop Drawings: Provide Shop Drawings/Coordination Drawings for all ceilings, which should include RCP and product details. Coordinate Wood Baffle layout and installation of wood baffles and suspension system components with other construction elements that penetrate ceilings or are supported by them, including light fixtures, HVAC equipment, fire-suppression system components, partition assemblies and all perimeter conditions.

1.7	PROJECT CONDITIONS

A.	Space Enclosure and Environmental Limitations: Do not install wood baffles until spaces are enclosed and weatherproof, wet-work in spaces is completed and dry, work above ceilings is complete, and ambient temperature and humidity conditions are being maintained at the levels indicated for Project when occupied for its intended use.

1.8	DELIVERY, STORAGE, AND HANDLING

A.	Delivery & Unloading: Coordinate crate sizes, weights, unloading options, and delivery schedule with manufacturer prior to fabrication. Deliver wood baffles and suspension system components to Project site in original, unopened packages and store them in a fully enclosed space where they will be protected against damage from moisture, direct sunlight, surface contamination, and other mistreatment.

B.	Acclimatization: Before installing wood baffles, permit them to reach room temperature and a stabilized moisture content (at least 72 hours) per AWI standards.

C.	Handling: Handle Wood Baffles carefully to avoid chipping edges or damaging units in any way.

D.	Protection:

1.	Personnel: Follow good safety and industrial hygiene practices during handling and installing of all products and systems, with personnel to take necessary precautions and wear appropriate protective equipment as needed. Read related literature for important information on products before installation. Contractor to be solely responsible for all personal safety issues during and subsequent to installation; architect, specifier, owner, and manufacturer will rely on contractor’s performance in such regard.

2.	Existing completed work: Protect completed work above suspension system from damage during installation of suspension system components.

1.9	EXTRA MATERIALS/WARRANTIES

A.	Extra Materials: Furnish extra materials described below that match products installed, are packaged with protective covering for storage, and are identified with labels clearly describing contents.

1.	Wood Baffles: Furnish quantity of full-size units equal to 2.0 percent of amount installed.

2.	Suspension System Components: Furnish quantity of each component equal to 2.0 percent of amount installed.

B.	Warranties: Provide owner with a (1) year warranty for material and workmanship on all installed products.

1.	Manufacturers: All materials, wood ceiling and suspension, shall be warranted for (1) one year for material and workmanship.

2.	Installer: All work shall be warranted for (1) year from final acceptance of completed work.

PART 2 – PRODUCTS

2.1	WOOD BAFFLE AND SUSPENSION SYSTEM

A.	General: The following manufacturer is basis of design:

1.	9Wood (www.9wood.com): 1500 Baffle.

B.	Or equal, as prior approved by architect.

2.2	WOOD BAFFLE

A.	Basis of Design: 9Wood, Inc. Wood Baffle, Series 1500

1.	Wood Baffles: 1500 Baffle, 1528-1

1)	Species: 	Cherry, Walnut, White Maple (Plain Sliced Veneer) or White Oak

(Rift Sliced Veneer)

2)	Member Size:	1 7/16” x 7 3/4" (net)

3)	Edge Profile:	Square, Edgebanded and endbanded

4)	Member Spacing:	<e.g. 12” On-Center>

5)	End-to-End Joints:	<if applicable: Closed butt-joint with removable locking biscuits>

6)	Baffle Lengths:	See architectural RCPs

7)	Fire Rating:	Class 1(A) Fire Rating

8)	Finish: 	Clear Interior Finish w/ Stain to Match Architects Control

9) 	Suspension:	Aircraft cable mounting brackets

2.3	METAL SUSPENSION SYSTEMS, GENERAL

A.	Aircraft Cable Suspension System: Provide stainless steel 1/16” 7x7 mil spec cable with factory-swaged fittings, rated for a minimum breaking strength of 3 times the operating weight, and conforming to ASTM A1023/A1023M.

B.	Height adjustment device: Provide load-rated cable grippers to allow height adjustment of system without using tools.

C.	Seismic Installation: Where applicable, install suspension system in accordance with ASCE7-10, including design load of anchors, consideration of seismic interaction effects, and range of motion.

PART 3 – EXECUTION

3.1	EXAMINATION

A.	General: Examine substrates and structural framing to which ceilings attach or abut, with installer present, for compliance with requirements specified in this and other sections that affect ceiling installation and anchorage. Do not proceed with installation until unsatisfactory conditions have been corrected.

3.2	PREPARATION

A.	Coordination: Furnish layouts for cast-in-place anchors, clips, and other ceiling anchors whose installation is specified in other Sections.

B.	Layout: Measure each ceiling area and establish the layout of Wood Baffles to balance border widths at opposite edges of each ceiling. Conform to the layout shown on reflected ceiling plans in accordance with wood ceiling manufacturer’s approved Shop Drawings.

3.3	INSTALLATION

A.	General: Install 9Wood, Inc. Interior Wood Baffle Style 1500 to comply with manufacturer's instructions and CISCA "Ceiling Systems Handbook."

B.	Attachments: Attach aircraft cable anchors from building's structural members per manufacturer’s instructions and in compliance with all local codes and regulations.

C.	Installation of Aircraft Cable: Install, align, brace, tie-off, mount, handle interferences, and space suspension in compliance with all local codes and regulations.

D.	Installation of Wood Baffle (Series 1000): Install Wood Baffles in accordance with manufacturer’s installation instructions and in compliance with all local codes and regulations. Install with undamaged edges and fitted accurately to suspension system runners and edge moldings. Scribe and cut baffles at borders and penetrations to provide a neat, precise fit, as required.

E. 	Levelness: Ensure that the face of installed Wood Baffles are level within 1/8” in 12’.

3.4	CLEANING

A.	General: Clean exposed wood surfaces of 9Wood, Inc. Style 1500 Wood Baffles. Comply with manufacturer's instructions for cleaning and touchup of minor finish damage. Remove and replace wood ceiling components that cannot be successfully cleaned and repaired to permanently eliminate evidence of damage.

END OF SECTION 09 54 26